Steps to program attiny85 using Arduino

Step 1: Connect Arduino uno and upload ArduinoISP code from File > Example > ArduinoISP, make sure attiny is not connected.

Step 2: Add the attiny board manager from settings if plugged in for first time. Paste the code “https://raw.githubusercontent.com/damellis/attiny/ide-1.6.x-boards-manager/package_damellis_attiny_index.json”

Step3: Select board from tools:

Step 4: Check the parameters of the chip.

Step 5: Connect the chip to Arduino as shown:

Step 6: Connect a 10uF or 20uF capacitor on Arduino pin GNDReset (Important), set arduino as ISP

Step 7: Burn the bootloader. A burn bootloader is necessary if the code does not upload to Arduino and shows a synchronous error.

Step8: Write any code and upload using programmer.

Voltage range of attiny85: 2.7 to 5.5V

Program storage: 8Kb

Connection:
Pin 1	------>	RST(P5)	------>	10(ard, SS {SPI})	
Pin 4	------>	GND	------>	GND(ard)	
Pin 5	------>	P0	------>	11(ard, MOSI {SPI})	
Pin 6	------>	P1	------>	12(ard, MISO {SPI})	
Pin 7	------>	P2	------>	13(ard, SCK {SPI})	
Pin 8	------>	VCC	------>	5V(ard)